csDronLink: Functions of the Dron class

The following table describes the functions of the Dron class in the current version of the csDronLink library. Many of them include the three parameters required to implement the non-blocking mode (bloquear, f, and param).

Basic commands

The essentials for the most elementary cycle: connect, arm, take off, land, and disconnect.

| public void Connecta(string mode, string connector = null) |  |
| --- | --- |
|  | Connects to the drone. The mode parameter can be “simulation” or “production”. In the first case, no connector is needed and it connects to “tcp:127.0.0.1:5763” at a speed of 115200. In the case of “production”, it connects to the port indicated by connector and the speed will be 57600. It also sends the telemetry requests via MAVLink protocol. |
| public void Takeoff(int altitude, Boolean blocking = true, Action<object> f = null, object param = null) |  |
|  | Takes off and climbs to the altitude specified by the parameter. |
| public void Land (Boolean blocking = true, Action<object> f = null, object param = null) |  |
|  | Commands the drone to land at its current position. |
| public void RTL(Boolean blocking = true, Action<object> f = null, object param = null) |  |
|  | Commands the drone to return to launch (home). |
| public void Arm() |  |
|  | Arms the drone |
| public void Disarm() |  |
|  | Disarms the drone |
| public void Send_RC_Override(List<ushort> channels) |  |
|  | Sends an RC command to the drone via MAVLink containing Roll, Pitch, Throttle, Yaw values between 1000 and 2000. Used for Joystick commands. |

Navigation & Movement

Commands to direct the drone to navigate or move to a specific geographic point.

| public void Navigate(string direction) |  |
| --- | --- |
|  | Makes the drone navigate in the specified direction. The options are: ‘North’, ‘South’, ‘West’, ‘East’, ‘NorthWest’, ‘NorthEast’, ‘SouthWest’, ‘SouthEast’. |
| public void GoToWaypoint(float lat, float lon, float alt, Boolean blocking = true, Action<object> f = null, object param = null) |  |
|  | Directs the drone to the specified geographic point. |
| public void Move(string direction, int distance, Boolean blocking = true, Action<object> f = null, object param = null) |  |
|  | Moves the drone the number of meters specified by distance in the given direction. The possible values for direction are: ‘Forward’, ‘Back’, ‘Left’, ‘Right’, ‘Up’, ‘Down’, ‘Stop’, ‘ForwardLeft’, ‘ForwardRight’, ‘BackLeft’, ‘BackRight’. |

Missions

Commands to load and execute missions. A mission is a sequence of waypoints that the autopilot will execute autonomously.

| public void UpLoadMission(List<(float lat, float lon)> mission) |  |  |
| --- | --- | --- |
|  | Loads the mission defined by the list of waypoints provided. You can also provide altitude |  |
| public void ExecuteMission(Boolean blocking = true, Action<object> OnWaypoint = null, Action<object> f = null, object param = null) |  |  |
|  |  | Executes the loaded mission. The drone must already be airborne before this command is issued. If a function is specified in the “OnWaypoint” parameter, the library will call that function each time a waypoint is reached, passing it the index of the waypoint reached. |  |
| public void CancelMission() |  |  |  |
|  |  | Sends the order to delete every waypoint and reset the mission counter. ATTENTION: it does not clear the message Handler waypoint queue. |  |

Scenarios

Scenarios include one inclusion geofence and several exclusion geofences representing obstacles in the flight area (there may be none).

| public void SetEscenario(List<List<(float lat, float lon)>> scenario) |  |
| --- | --- |
|  | The scenario is received as a list. Each position contains a fence representing an area. The first element of the list is an inclusion fence, representing the area (polygon) that the drone will not exit. The remaining elements of the list are exclusion fences representing obstacles within the inclusion fence that the drone cannot fly over. The scenario must have one inclusion fence (only one, as the first element of the list, with at least 3 waypoints) and a variable number of exclusion fences, which may be 0. An exclusion fence can be a polygon or a circle. In the first case, the fence must have at least 3 waypoints. If the fence is a circle, then it has 2 waypoints: the first identifies the center of the circle, and the latitude of the second waypoint represents the radius in meters. |
| public List<(List<(float lat, float lon)> vertices, string type)> Getscenario () |  |
|  | Gets the scenarios already loaded inside the drone and displays them on screen. |

Parameters Handling

| public List<float> ReadParameters(List<string> parameters) |  |
| --- | --- |
|  | Requests the value of the parameters whose names are provided in the list. Returns a list with the parameter values in the same order as the names. |
| public void Set_Parameter(string name, float value) |  |
|  | Sets the value of a single parameter, the name has to be one of the names present in the full parameter list. |
| public Dictionary <string, (float value, MAVLink.MAV_PARAM_TYPE type)> RequestAllParameters() |  |  |
|  |  | Tries to get all the parameters from the drone’s full parameter list. |
| public List <(string name, float value) ReadParametersViaListStream() |  |  |
|  |  | Returns the parameter list read with “RequestAllParameters()” as a list. ATTENTION: it is not guaranteed to get all of them. |

Flight Modes

| Private static uint MapModeToCode(string mode) |  |  |
| --- | --- | --- |
|  |  | It maps each mode written as a string (e.g. “LOITER”) to its corresponding unsigned integer to send via MAVLink (i.e. 5) |  |
| public void SetFlightMode(string mode, byte vehicleSysId = 1, byte gcsSysId = 255) |  |  |  |
|  |  | Sets the desired flight mode |  |

Heartbeat

| public void StartHeartbeatLoop(byte gcsSysId = 255) |  |
| --- | --- |
|  | Starts the heartbeat loop |
| public void StopHeartbeatLoop() |  |
|  | Stops the heartbeat loop |
| public void SendGcsHeartbeat (byte gcsSysId = 255) |  |
|  | Sends the GCS heartbeat packet |

Telemetry

Many informations of the drone are available through this section.

| public void SendTelemetryDataToClient(Action<byte, uint, List<(string name, float value)>> f) |  |
| --- | --- |
|  | Requests telemetry data from the drone. When the drone has a new telemetry packet, it calls the callback f and passes the packet as an argument. The packet is a list of tuples (name, value) containing the four data points. The field names used are: “Lat”, “Lon”, “Alt”, and “Heading”. |
| public void StopTelemetryData() |  |
|  | Stops the transmission of telemetry data. |
| public void RegisterTelemetry(MAVLinkMessage msg) |  |
|  | It oversees identifying message ID and sending it to the right telemetry handler |
| public void Telemetry_RequestList() |  |
|  | It sends the list of all telemetry requests to the drone for it to start sending data. Requested values for now are: Global Position, Local Position, Attitude, Battery Status, Scaled IMU, Heartbeat. |
| public void Telemetry_<NameofthePacket>(MAVLinkMessage msg) |  |
|  | It is a series of very similar functions that take data from the MAVLink message, and they place them in a List, after proper rescaling (if necessary). In the end they invoke a function which is the function in the application that handles the telemetry and stores or displays the values. There is one function for each packet listed above. |

Change Vel (new Miscellaneous)

| public void SetGuidedMode() |  |
| --- | --- |
|  | Sets the drone to “Guided” mode. |
| public void ChangeHeading(float newHeading, Boolean blocking = true, Action<object> f = null, object param = null) |  |
|  | Rotates the drone clockwise until it is oriented according to new Heading. |
| public void ChangeSpeed(int speed) |  |
|  | Changes the navigation speed. |

Move Indoor

| public void MoveIndoor(float x, float y, float z, Boolean heading = true, Boolean blocking = true, Action<object> f = null, object param = null) |  |
| --- | --- |
|  | Moves the drone of a relative displacement (with respect to where the drone is now) in x, y, z. |
| public void MoveIndoor_Abs(float x, float y, float z, Boolean heading = true, Boolean blocking = true, Action<object> f = null, object param = null) |  |
|  | Moves the drone to an absolute position in x, y, z (the origin is the takeoff point). |